October 2019

SWIFT Suite 1.8.2 Update

What is SWIFT Suite?

This is a collection of firmware updates and utilities for our devices, such as IP gateways, option boards and media docks.

What’s new in 1.8.2?

A100/A200 IP Gateways

Support for VOX and MTM control stations

As part of our strategy to integrate legacy radios and digital non-MOTOTRBO platforms with MOTOTRBO radio infrastructure, we are releasing new versions of the firmware for IP Gateways to support VOX-capable radios and MTM 5400 and 5500 control radios to connect Tetra channels. Supported Tetra features: radio IDs, group calls.

Requirements: TRBOnet 5.4.0.2199 or newer.

Option Boards

Voice Recording

You can now use TRBOnet option boards for voice recording. With the storage capacity of 256Mb allocated for audio files, you can have up to 200 hours of continuous voice recording. Motorola’s GOBs are also capable of storing voice recordings, but they are limited to 8Mb, or up to 5-6 hours of continuous recording.

User Authentication

In an attempt to prevent radio thefts and losses, system providers and owners may be interested in an efficient mechanism enabling dispatchers to keep track of radios. TRBOnet 5.4.0.2199 and newer in combination with the latest SWIFT program for option boards allows radio users to get registered with the dispatcher system when radios are checked out to them. A radio subscriber does not have to punch in his or her password to sign in, instead he or she uses a dedicated iBeacon-based transmitter as a security token that in fact transmits authentication information via Bluetooth into the dispatcher console. After successful authentication, the user name will be displayed next to the radio ID in the dispatcher console. The token must be visible to the radio all the time the user carries it. There are a variety of options you can select to adapt this feature to your needs. For instance, the option board can “lock” the radio if the iBeacon token is not in range.  When the user returns (checks in) the radio, it is no longer associated with this user.

Requirements: TRBOnet 5.4.0.2199 or newer, a Bluetooth-capable DP/DM4000 radio with an option board.

Scenarios

The event handling system for option boards has become even more flexible and versatile. Based on Rules, the option board software did not allow for repetitive actions or custom event handling procedures. The present version of the Suite introduces Scenarios, user-defined sets of actions that can be started within any rule. We have also added the ability to loop a task sequence. As an example, it can be used for a periodic check of the radio’s orientation after a rule has been triggered.

Mass OTA Configuration

If you have tens or hundreds of radios equipped with option boards, it might be a bit annoying to connect them to your computer one by one to make minor changes in the configuration. The new Suite simplifies this routine actions by sending the updated configuration file over a radio channel to specific radios or a pool of radio IDs.

Note: this feature does not allow for remote firmware update.

Dynamic Сhannel Selection

The option board is able to automatically select one of the predefined channels depending on its location that can be expressed as a geofence region or proximity to an iBeacon with a specific ID or ID range. The feature can be beneficial to roaming radio users who can have their radios automatically select a radio channel when they enter the coverage area of another radio system.